摘     要

小波分析是近10年来迅速发展起来的新兴学科，它同时具有理论深刻和应用十分广泛双重意义。它是泛函分析、Fourier分析等分析学科综合而发展起来的新兴技术。Mallat将计算机视觉邻域内多尺度分析的思想引入到信号处理邻域，提出了多分辨分析的概念，用多分辨分析来定义小波，并给出了和FFT相对应的快速小波算法，并将它应用于图像分析和压缩重构，开创了小波理论在信号处理邻域的应用，使得小波分析与小波变换成为信号处理学科的一个新的分支，并得到迅速发展。小波和滤波器组联系的建立，使得M带小波变换广泛的应用在信号处理中。

作为小波分析的应用之一，对于图像压缩方法的研究，得到了更进一步的发展。本论文研究了小波分析在图像压缩中的应用，主要贡献有以下四点：

- 针对信号的子带分析中有限长滤波器的应用，从Mallat算法完全重构的思想以及因线性滤波而使得原图像数据点数增加的缺点，给出了一种二带小波变换和M带小波变换滤波器长度为偶数时的边界延拓算法，使得重构图像在分解层数较高时也能达到相当高的峰值信噪比。

- 结合M带小波分解的特性，以及均匀量化加上熵编码在低比特率下接近最佳熵约束标量量化的量化特性，提出一种与人类视觉特性有关的量化方案，并较好的实现了在满足视觉感知无失真前提下的图像压缩编码。

- 针对原图像方块编码所带来的不良的分块边界效应，并结合M带小波变换优良的分频特性，提出一种子带方块编码的图像压缩方法，有效地解决了子带方块编码所带来的两个问题1）如何选取合适的子带2）对选取的子带如何确定分块的大小；同时有效地消除了分块边界效应。

- 零树编码以其高压缩比得到了人们的重视，但其搜索编码所涉及的链表的插入和删除等操作，以及解码阶段逐节迭代恢复重要系数给其的推广带来不便，因而给出一种简单的实现方法使其时间复杂度和空间复杂度降低。

关键词   小波分析，Mallat算法，M带小波变换，滤波器组，边界延拓，图像压缩，量化，方块编码，零树编码

# Image compression Through Wavelet Transform Coding

## Hou    Biao

Department of Mathematics

Northwest University

Xi’an,710069

# Abstract

The development of wavelets during the past decade has been closely associated with advances in signal analysis.Both the mathematical theory and the applications seem to have benefitted from this synergy,it is also a new technique comprehending Functional analysis,Fourier analysis and so on.Mallat introduces the thought of multiresolution of computer vision to signal analysis,and developes the important concept of multiresolution analysis.He defines wavelet by multiresolution analysis(MRA),a fast wavelet algorithm(FWT) corresponding with FFT is proposed,it is widely used in image analysis and image compression and reconstruction.A major impact on signal processing has been made.Consequently,wavelet analysis and wavelet transform have been a new branch of signal processing,and are made rapidly progress.A concise,yet thorough,introduction to the discrete filter banks used in signal analysis and their relation to discrete wavelets is given by Vetterli,which makes M_Band wavelet useful in signal processing.

Image compression,being an application of wavelet analysis,has been made further progress.This paper put an emphasis on image compression and reconstruction with wavelet analysis,four major contributions are made in the paper:

Firstly,a bound extended algorithm of two_band and M_band wavelet transform is proposed in the case that the length of FIR is even.FIR is 1) two_band wavelet with orthogonal and compactly support properties,i.e.,Daubechies wavelet.2)M_band wavelet with linear_phase and perfect_reconstruction properties.Higher PSNR of the reconstrued image may be achieve in bigger wavelet decomposition scale.

Secondly,an image compression methord is proposed based on M_band wavelet using perceivable entropy coding which exploits human visual characteristics and quantization characteristics of methord combining uniform quantization and entropy coding approaching optimal entropy_restrained scale quantization in lower bit rate.

Thirdly,a subband block coding methord is proposed exploiting frequency characteristics of M_band wavelet transform,and two consequent problems are efficiently solved,i.e.,1)How to select proper subband.2)How to determine the size of block of selected block.Meantime,bound effect due to block is efficiently removed.

Fourthly,a simple implement of EZW is proposed.It discards insert and delete of link list,and simplifies the process of recovering the important coefficient in EZW ,which makes time complexity and space complexity lower.

Key words:Wavelet analysis,Mallat algorithm,M_Band wavelet   transform,Filter banks,bound extended,image compression,quantization,block coding,EZW

目    录

Contents

- 小波分析及其发展……………………….…………..(1)

  - 小波分析的发展历史……………………………………….(1)

  - 傅立叶变换存在的问题…………………………………….(2)

  - 小波分析的引入………………………………………...….(3)

1.4  多尺度分析…………………………………………………(4)

1.4.1 一维多尺度分析…………………………………..……(4)

1.4.2 二维多尺度分析…………………………………..……(4)

1.5  离散正交小波变换算法（Mallat算法）………………….(5)

1.5.1 一维信号的Mallat算法………………….……….……(6)

1.5.2 二维图像的Mallat算法………………….……….……(7)

  - 紧支撑正交小波基…………………...……………….……(8)

- 二带到M带小波变换的扩展…………………..(10)

  - M带多尺度分析…………………………………………..(10)

  - 一维信号的M带离散正交小波变换…………………..….(10)

  - 二维图像的M带离散正交小波变换…………………..….(12)

- 图像压缩与子带编码………..……………….……(14)

  - 信号、图像与小波……………………………………….(14)

  - 图像质量评价标准………………………………………(14)

  - 信息熵编码方法…………………………………………(15)

    - Huffman编码………………………………….……(15)

    - 游程编码……………………………………………(16)

    - 算术编码……………………………………………(17)

  - 预测编码…………………………………………………(17)

  - 基于线性变换的压缩系统………….………………….…(19)

    - 线性变换………………………………………….…(20)

    - 量化………………………………………………....(20)

    - 熵编码…….……………………………….………..(20)

  - 基于小波变换的图像压缩编码……………….………….(20)

第四章  小波变换边界延拓算法研究…………..….….....(22)

4.1  有限长度滤波器的边界延……………...……………..…(22)

  - 二带离散正交小波分解、重构的边界延拓算法.………..(23)

  - M带离散正交小波分解、重构的边界延拓算法.…….…(26)

第五章  基于M带小波的图像编码方法.………..……..(28)

  - 序言.……………………………………………………...(28)

  - 小波的正则性.…………………………………………...(28)

  - 基于M带小波图像压缩编码.………………….…….…(28)

    - 细节的量化.…………………………………………(28)

    - 模糊像的处理.………………………………….……(30)

    - 编码.…………………………………………….….(30)

    - 实验结果.…………………………………………...(31)

    - 结论与讨论.…………………………………………(32)

  - M带小波滤波器的选取…………………………………(32)

- 基于M带正交小波的方块编码………………..(34)

6.1   序言….……………………………………………….…..(34)

6.2   方块编码的基本方法……………………………………(34)

6.3   M带小波分解的子带方块编码…………………………(35)

6.4   实验结果…………………………………………………(38)

第七章  基于零树假设的图像编码……...…………..……(40)

7.1   序言……………………………………….……………...(40)

7.2   图像小波分解的零树表示…………………………....…(40)

7.2.1  零树假设的提出….…………………………………..(40)

7.2.2  零树编码的基本思想…………………………………(41)

7.3   基于零树假设的算法分析………………………………(42)

7.4   编码及解码算法…………………………………………(43)

7.4.1  编码算法…………………………………………….(43)

7.4.2  解码算法…………………………………………….(43)

7.5   实验结果…………………………………………………(44)

  - 结论与讨论………………………………………………(46)

总结…………………………………………………………………...(48)

参考文献………………………………………………………….…(49)

致谢…………………………………………………………………...(52)